اعتبارسنجی روش اندازه‌گیری کافئین در نوشیدنی قهوه با کمک طیف سنج تحرک یونی

مرضیه معین فرد *1، شیما تقوی2، پانیذخالوکرمانی3، علی شریف4

1- استادیار گروه علوم و صنایع غذایی، دانشگاه فردوسی مشهد

2- دانشجوی کارشناسی ارشد علوم و صنایع غذایی، دانشگاه فردوسی مشهد

3- دانشجوی کارشناسی ارشد علوم و صنایع غذایی، دانشگاه فردوسی مشهد

4- استادیار گروه علوم و صنایع غذایی، دانشگاه فردوسی مشهد

moeenfard@ferdowsi.um.ac.ir

چکیده

بيان مسأله: کافئین با نام شیمایی 7،3،1-تری متیل زانتین، یک آلکالوئید پورینی است که در قهوه، به مقدار زیاد یافت می‌شود. روش‌های متعددی جهت آنالیز کافئین در قهوه پیشنهاد شده که اکثر آن‌ها بر آنالیز عصاره با کروماتوگرافی مایع یا گازی استوار است که بسیار دقیق و حساس هستند. با این وجود، هزینه‌بر بودن، زمان آنالیز طولانی و مصرف مقادیر زیاد حلال آلی نیز از معایب این روش‌ها به شمار می‌روند. در نتیجه استفاده از یک روش سریع، دقیق و حساس که بتواند کافئین را در مدت زمانی کوتاه تعیین مقدار کند، از اهمیت بالایی برخوردار خواهد بود. در این میان، اسپکتروسکوپی تحرک یونی از جایگاه ویژه‌ای برخوردار است.

هدف پژوهش: هدف از این پژوهش اعتبار سنجی روش اندازه‌گیری کافئین در قهوه با کمک طیف سنج تحرک یونی است.

روش و چگونگي انجام پژوهش: استاندارد کافئین در محلول متانول 10% و در محدوده ppm 100-1 تهیه شد و سپس پارامترهای ضریب تعیین (R2)، دقت و تکرارپذیری (CV %)، صحت (Recovery%) ، حد تشخیص (LOD) و حد اندازه‌گیری (LOQ) مورد مطالعه گرفت. استخراج کافئین از نمونه‌های واقعی شامل نوشیدنی قهوه، نوشیدنی هسته خرما و نمونه‌ شاهد با کمک محلول کاریز 1 و 2 انجام گرفته و عصاره‌های حاصل با روش پیشنهادی آنالیز شدند.

يافته ها و نتيجه گيری: به دلیل اشباعیت سیستم در غلظت‌های بالای استاندارد، نمودارخطی در محدوده‌ی غلظتی mg/L 5/17-5، (پنج نقطه) جهت اعتبارسنجی و آنالیز نمونه‌ها مورد استفاده قرار گرفت. پاسخ دستگاه در این محدوده خطی و در حدود 95/0گزارش شد. مقادیر حد تشخیص و اندازه‌گیری به ترتیب mg/L 6/0 و 9/1 بدست آمد. درصد ضریب تغییرات در یک روز و بین روزهای مختلف برای غلظت‌های کم (mg/L 5)، متوسط (mg/L 10) و زیاد استاندارد (mg/L 5/17) بین 1/8–9/0% متغیر بود بطوریکه بیشترین درصد ضریب تغییرات برای غلظت‌های کم استاندارد و کمترین درصد ضریب تغییرات برای غلظت‌های بالای استاندارد به دست آمد. روش استخراج نیز منجر به دریافت مقادیر قابل قبول درصد بازیابی (بیشتر از 92%) شد. مقدار کافئین در قهوه هسته خرما و قهوه بدون کافئین، کمتر از حد تشخیص گزارش گردید. نمونه‌های قهوه جوشیده (100% قهوه عربیکا) و قهوه ترک به ترتیب با مقادیر mg/L 38±688 و 17±625 بیشترین مقدار کافئین را داشته و نمونه‌های قهوه‌ که با 50% هسته خرما جایگزین شده بودند، کمترین مقدار کافئین (mg/L 24±410) را به خود اختصاص دادند. نتایج این تحقیق نشان داد که طیف سنج تحرک یونی به عنوان یک روش صحیح، دقیق، سریع و سازگار با محیط زیست، می‌تواند جهت پایش پیوسته کافئین در انواع نوشیدنی قهوه مورد استفاده قرار گیرد.

کليد واژه ها: کافئین، طیف سنج تحرک یونی، اعتبار سنجی روش، قهوه، قهوه هسته خرما

Abstract

Method Validation for Caffeine Quantification by Ion Mobility Spectrometry

Marzieh Moeenfard *1, Shima Taghavi 2, Paniz Khaloo Kermani 3, Ali Sharif 4

1- Assistant professor, Department of Food Science and Technology, Ferdowsi University of Mashhad

2- Master student, Department of Food Science and Technology, Ferdowsi University of Mashhad

3- Master student, Department of Food Science and Technology, Ferdowsi University of Mashhad

4- Assistant professor, Department of Food Science and Technology, Ferdowsi University of Mashhad

moeenfard@ferdowsi.um.ac.ir

‌   Statement of Problem: Caffeine, also known as 1,3,7-trimethyl xanthine, is a purine alkaloid found in coffee in large quantities. Various methods have been proposed for the analysis of caffeine in coffee, most of which are based on the analysis of the extract by liquid or gas chromatography, which are highly sensitive and accurate. However, these techniques are time consuming and suffer from using large amounts of organic and expensive solvent consumption, as well. Therefore, using a fast, accurate and sensitive method based on ion mobility spectrometry (IMS) which can identify and quantify caffeine in coffee brews over a short period of time will be of great interest.

Research Purpose: The main objective of the present study was to validate the method of caffeine quantification in coffee brews using ion mobility spectrometer.

Research Method: Caffeine standards solutions were prepared in 10% methanol (v/v) in the range of 1 to 100 mg/L. Validation parameters, including linearity (expressed as coefficient of determination, R2), precision (expressed as coefficient of variation, %CV), accuracy (expressed as percentage of recovery), limit of detection (LOD) and limit of quantification (LOQ) were determined. Caffeine was extracted from real sample including coffee brews, date coffees and control using Carrez solutions I and II and clarified extracts were analyzed using validated method.

Results and Conclusion: Due to the saturation of the IMS cell at high standards concentration, a calibration curve was plotted in the range of 5 to 17.5 mg/L (five points). The calibration curve was linear in the studied range (5-17.5 mg/L) with high coefficient of determination (R2 ≥ 0.95). Regarding LOD and LOQ the values of 0.6 and 1.9 mg/L were obtained, respectively. Precision of the analytical method was evaluated through intra- and inter-day precision assays of standard samples at three caffeine concentration levels (5, 10 and 17.5 mg/L) which varied from 0.9 to 8.1%. The highest and lowest precisions were obtained from low and high standard concentrations, respectively. The accuracy of the analytical method was studied through recovery test as the extraction approach resulted in acceptable recovery (≥ 92%). The amount of caffeine in date and decaffeinated coffees were below the detection limit. In general, boiled (100% Arabica) and Turkish coffees were the most concentrated brews in terms of caffeine content (688±38 mg/L and 625±18 mg/L, respectively). While, boiled coffee containing 50% of roasted date pits powder had the lowest contribution to caffeine intake with value of 410±24 mg/L. The results of the present study proves that ion mobility spectrometer as an accurate, fast and environmentally friendly analytical technique can be used for continuous monitoring of caffeine in coffee beverages.

Keywords: Caffeine, Ion Mobility Spectrometer, Method Validation, Coffee, Date Coffee